Components Of Robot

- IR Array – 5 channels.

- L298N Motor Driver Module.

- IR Object Detector.

- MPU6050 Gyro & Accelerometer Module.

  - Very Low accuracy.

  - Bugs due to: Library or Module.

SW Specs.

- Sensors:

  - Type: Analog or Digital.

  - Number of Sensors.

- Pins.

- Bootloader (FOTA).

- Internal Debugger.

- Emulators (Programming debugging, energy measurements).

Arena Structure

- 2 Pick-ups (GUIs).

- 2 Rows * 5 Segments.

- 1 AGV.

- 1 Transmission Line.

Process of Project

RFQ (Request For Quotation)

An Auto Guided Vehicle (AGV) manages a factory consists of:

- 2 Production Lines.

- A warehouse consists of:

  - 2 Rows of roughs.

  - Each one has 5 roughs, each rough has different raw material.

The AGV takes the order wireless and brings the rough of the required raw material to the commander production line.

CRS (Customer Requirement Specs)

- 2 Master controllers for each production line.

- 1 GUI for each production line.

- 1 Slave microcontroller for the AGV.

- Masters send commands with wireless communication.

- The AGV navigates to the required position.

- The lifter mechanism goes up until sensing the rough and lift it 1 cm.

- The AGV goes back to the transmission line intersection and down the lifter.

- The AGV navigates to the commander production line.

- When it reaches the goal, it waits until the rough is lift-off it.

SRS (SW Requirement Specs.)

A SW performs the following:

- GUIs, warehouse points, charging ports, and waiting points are defined in diff. arrays.

- The AGV performs:

  - if (level < 20 %), Send LOW-LEVEL battery and NOT-AVAILABLE state and indicator lamp of availability RED.

POSITION (X1, Y1) and COMMANDER_GUI (X2, Y2) are in ROM for remembering it is destination when restarting.

  - If battery is OK, check for previous destinations in EEPROM.

If exists and pressure sensor:

    - detects rough, it goes to previous destination in EEPROM.

    - does not detect rough, it reset the values in EEPROM, send a request to the COMMANDER_GUI to re-request it’s order if still needing it, check battery conditions to recharge or go to waiting room.

  - send AVAILABLE state to masters and indicator lamp of availability GREEN.

  - Sends AVAILABLE state to masters.

- When Battery level reaches some value, an ISR sends low battery and AGV goes to an available charging port in order priority.

- Master goes to sleep until an interrupt for an order from GUI (LabVIEW + VISA).

- Check availability of AGV (low battery, under maintenance, ….)

  - If (available) Send POSITION (X1, Y1) and COMMANDER_GUI (X2, Y2) to the AGV wirelessly (NRF24L01+).

  - If (not available), store in the QUEUE until AGV sends availability status.

- The AGV get: POSITION (X1, Y1) and COMMANDER_GUI (X2, Y2).

- Navigates to (X1, Y1) and continuously send it’s position to master for visualization.

  - If not reached and remains at the same place for SOME TIME, do ALERT!.

    - Maintenance is needed, and when done

  - If reached, lifter UP, when pressure sensor detects rough, lift for 1 cm, go back to the intersection point (X1, 1), lifter DOWN (reverse direction).

- Navigates to COMMANDER_GUI (X2, Y2).

  - If not reached and remains at the same place for SOME TIME, do ALERT!.

  - If reached, Wait until pressure sensor detects nothing (a human picked the rough), navigate to WAITING_ROOM

HLD (Hard Level Design)

- Module for IR Array.

Input: NO

Return:  1 or 0 for each channel

- Module for MPU.

Input: NO

Return: yaw angle (-360 ~ 360).

- Module For Motors direction Driver (L298).

Input: direction. (FWD, RIGHT, LEFT, BACK).

Return: NO (may we get a return indicating an error; a motor is not working).

- Module for IR object detector.

Input: NO.

Return: 1 or 0 (indicates object exists or not).

- Module For Lifter Mechanism.

Input: UP, DOWN.

Return: NO.

- Module for Pressure Sensor.

Input: UP, DOWN.

Return: NO.